# Lockout–Tagout (LOTO) Standard Operating Procedure

Training materials  ·  EHS  ·  Owner: R. Karthik  ·  8 Apr 2026  ·  v1.0  ·  EN/TE/HI

The six-step energy-isolation procedure to be followed before servicing any machine that can start unexpectedly.

#### Why LOTO

Lockout–Tagout prevents the accidental start-up or release of stored energy while a machine is being serviced. It is mandatory for any maintenance, cleaning, or clearing of a jam.

#### The six steps

- Prepare. Identify all energy sources — electrical, pneumatic, hydraulic, mechanical, thermal.

- Shut down. Stop the equipment using the normal stopping procedure.

- Isolate. Operate the disconnect/valve to isolate each energy source.

- Lock & tag. Apply your personal lock and a tag stating who, why and when.

- Release stored energy. Bleed pressure, discharge capacitors, block suspended parts.

- Verify. Attempt a normal start to confirm zero energy, then return controls to off.

#### Group lockout

When several people work on one machine, each applies their own lock to a group lockout box. The machine cannot be re-energised until the last lock is removed.

#### Golden rules

- One lock, one key, one person — never share or remove another's lock.

- If in doubt, do not proceed; call your supervisor.

Karya Vaani · Knowledge Center · Customer Plant — controlled document, v1.0