# Forklift & Material Handling Competency Pack

Training materials  ·  Logistics  ·  Owner: Naveen Kumar  ·  28 Feb 2026  ·  v1.0  ·  EN/TE/HI

Safe operation of forklifts and manual handling techniques, with the certification requirements to operate.

#### Who may operate

Only associates holding a valid in-plant forklift licence, issued after this training and a practical assessment, may operate a forklift. Licences are renewed annually.

#### Pre-use inspection

- Check tyres, forks, hydraulics, horn, lights, seat belt and the load chart.

- Report any defect and tag the truck out of service.

#### Safe operation

- Wear the seat belt; sound the horn at intersections and blind corners.

- Never exceed the rated load; keep the load low and tilted back while travelling.

- Keep pedestrians clear — maintain the marked forklift–pedestrian segregation.

- Travel in reverse when the load blocks your view.

#### Manual handling

For manual lifts: assess the load, bend the knees not the back, keep the load close, and get help or a trolley for heavy or awkward items. Report any strain early.

Karya Vaani · Knowledge Center · Customer Plant — controlled document, v1.0